Step 1:

Step 2: Setting up the GWINSTEK AFG-3051

- Turn the device ON using the power button.

- Insert the USB Drive containing the PESMOS data.

- Select the USB option in the AFG and rotate the knob to go to the selected data.

- Tap Main Output.

Step 3:  Construction of the Earthquake emulator (Shaker Table)

- A   4 Ω, 6’’ woofer is taken.

- A cylindrical dielectric (fiber-plastic) open on both ends of height 1.5’’ is taken.

- The cylinder is placed over the diaphragm and fixed with strong glue such that there remains no air gap between the cylinder end surface and the diaphragm.

- A thin (0.05mm) smooth surface (e.g. , SUNMICA) is placed over the other open end of the cylinder and fixed with the dielectric

- Wires are soldered over the two terminals of the woofer through which the signal is to be applied.

Step 4: Assembling the circuit for Earthquake Emulation.

- The 5 volt output of the Arduino board is connected to the Vcc pin of the MPU 6050 module.

- The ground of the Arduino board is connected to the ground of the MPU 6050 module.

- The SCL (Serial clock) of the MPU 6050 module is connected to the SCL pin of the Arduino board.

- The SDA (Serial data) of the MPU 6050 module is connected to the SDA pin of the Arduino board.

Step 5: Source code for seismic wave detection

- The source code was developed in the Arduino IDE. The references to the libraries called in the code are provided in the link  https://www.arduino.cc/en/Reference/Wire

- For understanding the code, basic knowledge of I2C communication protocol is necessary.

- The datasheet and register map of MPU 6050 module is provided below :-

****You can avail the code at ****

Step 6: The MACRO used for calculations and graphing (in MS-EXCEL)

^^ For reference on the MACRO please read -

- Using this Macro and the modifications in the code made for implementing the macro, we can plot the Graph by using Line Graph option from the toolbar.